Curriculum Learning – Group K

This is the homework assignment for Curriculum Learning for group K.

We will show the reward function used to compute the factorized reward. The different rewards used to compute the final one are collision avoidance, progress toward the goal, smooth steering, time to target and goal reaching.

We uploaded the turtlebot3_world.py file, in which the goal sampling curriculum is also implemented.

## Reward Function

def _compute_reward(self, observations, done):

# --- 1. State and Distance ---

x, y, _ = self._get_robot_pose()

dist = self._distance_to_goal(x, y)

if self.prev_dist is None:

self.prev_dist = dist

# --- 2. Compute factorized components ---

# A. Progress Reward: based on distance difference (Reward Shaping)

# If diff > 0 the robot got closer, if < 0 it got farther

diff = self.prev_dist - dist

r_progress = self.w_progress * diff

# B. Safety / Collision Avoidance

# Compute min valid laser scan

valid_scan = [o for o in observations if not numpy.isinf(o) and not numpy.isnan(o)]

min_scan = min(valid_scan) if len(valid_scan) > 0 else 10.0

r_collision_avoid = 0.0

threshold_safe = 0.3 # meters

if min_scan < threshold_safe:

# Linear penalty: the closer it is, the more negative the reward

r_collision_avoid = -self.w_collision * (threshold_safe - min_scan)

else:

r_collision_avoid = 0.1 * self.w_collision  # Small reward for being in a safe zone

# C. Smooth Steering

# Penalize if last action was not FORWARDS

r_smooth = 0.0

if self.last_action != "FORWARDS":

r_smooth = -self.w_smooth

# D. Terminal Rewards

r_terminal = 0.0

if done:

if self.reached_goal:

r_terminal = self.terminal_goal

elif self.steps >= self.max_steps_per_episode:

r_terminal = self.terminal_timeout

else:

# If done is True but not goal or timeout, it's a collision

r_terminal = self.terminal_crash

# E. Time Rewards

r_cumm_time = self.r_time * self.steps

# --- 4. Final computation and update ---

reward = r_progress + r_cumm_time  + r_smooth + r_collision_avoid + r_terminal

# Update cumulative reward components

self.cum_r_progress += r_progress

self.cum_r_time += self.r_time

self.cum_r_smooth += r_smooth

self.cum_r_collision_avoid += r_collision_avoid

self.cum_r_terminal += r_terminal

# Update state variables for next step

self.prev_dist = dist

self.prev_action = self.last_action

# --- 5. Logging structure ---

self.last_reward_components = {

"r_progress": float(r_progress),

"r_time": float(self.r_time),

"r_smooth": float(r_smooth),

"r_collision_avoid": float(r_collision_avoid),

"r_terminal": float(r_terminal),

"dist": float(dist),

"min_scan": float(min_scan),

"goal_x": float(self.goal_xy[0]),

"goal_y": float(self.goal_xy[1]),

"r_total": float(reward),

"cum_r_progress": self.cum_r_progress,

"cum_r_time": self.cum_r_time,

"cum_r_smooth": self.cum_r_smooth,

"cum_r_collision_avoid": self.cum_r_collision_avoid,

"cum_r_terminal": self.cum_r_terminal,

"cumulated_reward": self.cumulated_reward,

}

self.cumulated_reward += reward

self.steps += 1

print("Step Reward: {:.2f} | Components: {}".format(

reward, self.last_reward_components))

return reward

## Tests

### Run 1

Deep learning model

Rewards weights and values

| Reward | Value |
| --- | --- |
| w_progress | 20.0 |
| w_collision | 2.0 |
| w_smooth | 0.15 |
| terminal_goal | 100.0 |
| terminal_crash | -25.0 |
| terminal_timeout | -10.0 |
| r_time | -0.05 |

Performance metrics

### Run 2

Deep learning model

Rewards weights and values

| Reward | Value |
| --- | --- |
| w_progress | 20.0 |
| w_collision | 10.0 |
| w_smooth | 0.1 |
| terminal_goal | 150.0 |
| terminal_crash | -100.0 |
| terminal_timeout | -10.0 |
| r_time | -0.05 |

Performance metrics

### Run 3

Deep learning model

Rewards weights and values

| Reward | Value |
| --- | --- |
| w_progress | 20.0 |
| w_collision | 5.0 |
| w_smooth | 0.1 |
| terminal_goal | 100.0 |
| terminal_crash | -50.0 |
| terminal_timeout | -10.0 |
| r_time | -0.05 |

Performance metrics

### Run 4

Deep learning model

Rewards weights and values

| Reward | Value |
| --- | --- |
| w_progress | 20.0 |
| w_collision | 5.0 |
| w_smooth | 0.1 |
| terminal_goal | 100.0 |
| terminal_crash | -50.0 |
| terminal_timeout | -10.0 |
| r_time | -0.05 |

Performance metrics